# High-Resolution Optical Spectrometer (HROS): SRD Description

- General description

This instrument will provide high spectral resolution data in the optical range, suitable for detailed study of stars, quasars, and planet radial velocity programs.

- Wavelength range

0.31µm- 1.1µ, goal 0.31-1.3µm

Discussion: working into the infrared is scientifically valuable, but CCD’s cut off at 1.1µm.  New detectors are needed to reach the long wavelength limit.

- Field of view

10 arcsec

- Slit length

A slit length of 5 arcsec is needed, with this separation between orders

- Image quality

Image quality in the detector should be no worse than 0.2 arcsec FWHM

- Spatial sampling

Sampling should be no coarser than 0.2 arcsec

- Spectral resolution

R=50,000 (1 arc-sec slit, image slicer for R≥90,000)

Options: single slit, fiber feed for multiplexing

- Sensitivity

Must maintain 30m aperture advantage over existing similar instruments.

# CoDR Comments

Mostly technical comments for both the Santa Cruz and Colorado versions of HROS.

- Santa Cruz design concerns:

  - Mosaic grating fabrication/tolerancing

  - Collimator mirror (cost risk)

- Colorado design concerns:

  - No mechanical drawings or design

  - As designed requires a 140 m3 vacuum container

  - 32 large-format CCDs/dewars

# MODRES option

The possibility of adding a fiber feed to the Santa Cruz HROS design (MTHR) that would provide multi-object spectroscopic capability over the full delivered field (~20) was proposed in the original study for this instrument in 2002. The concept is for a fiber positioner (not designed as part of the MTHR effort) to be placed in the TMT focal surface with the fibers feeding a collimator then using the MTHR cross-dispersing elements, camera and detectors. The focal-plane scale is such that each “fiber” will be a multi-fiber hexpack image slicer. With the nominal MTHR cross-disperser grating, the spectral resolution for 1 fiber bundles would be R=2300. The idea would be to replace some or all of the WFOS capability with this add-on to MTHR. The limitation with respect to any of the WFOS designs under consideration would be light loss for a fiber system compared to a focal-plane slit mask design and possibly less effective sky subtraction with the fiber system.

The fiber positioner and associated atmospheric dispersion compensator and possibly field flattener/corrector designs have not been pursued as a part of any of the TMT studies to date.

In the MTHR design, the additions to the baseline high-dispersion elements are a fiber –feed collimator (moveable) and a pickoff mirror (moveable) and, if higher dispersion is required for the MOS/fiber mode the ability to swap out cross-dispersers. For the single-arm MTHR, the additional cost was estimated at $2.5M.

Questions to be addressed:

- Science case needs to be considered more deeply

- Science-based requirements need to be developed

- Operational model(s) need to be developed

- Fiber positioner/feed required conceptual design and costing.

# Required SAC reading

http://project.tmt.org:8080/docushare/dsweb/Get/Document5174/MTHR_feasibility_study.pdf

There is a well-developed broad science case here (40+ pages).